PORT SAN LUIS BOAT PUT AWAY SHEET

(Revised 5/23/14)

PLEASE BE SURE TO  FILL OUT AND KEEP THIS SHEET IN

BINDER ON BOAT EACH TIME YOU USE IT.

Mainsail cover is on, main halyard is removed from sail and secured to

lifeline, and mainsheet coiled and hung up to dry.					(Check)____

Self-furling genoa is fully wound up with at least three additional wraps

of the genoa sheet around it.  Both genoa sheets are then pulled tight

and attached by clove hitch to both winches.						(Check)____

Outboard motor fuel cap and vent are closed. It is tilted up and fully out

of water.  Leave  fuel control lever centered as you found it.				(Check)____

White mooring cable is hooked to mooring pennant metal fitting (larger line)

and  mooring ball line(smaller line) is cleated to foredeck cleat, leaving

two feet of slack										(Check)____

Tiller cover is on and centered with cleated line						(Check)____

Two winch handles are put away in ice chest					           (Check)____

Any bilge water is sponged up								(Check)____

Cabin hatch and two cockpit lockers are closed and locked, & forward deck

hatch is closed & secure									(Check)____

Boat deck and cockpit are mopped  and everything is neatly stored and

shipshape											(Check)____

Note any discrepancies________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

Skipper’s printed last name_______________________________________  Date__________